AFRIK CURLEW

GENERAL

bureau veritas | hull mach tug fire fighting ship 1 – water spraying unrestricted navigation

delivery date: 			july 2005

imo number: 							9327944

DIMENSION

length over all: 							47.40 m

beam over all: 							12.60 m

maximum draught: 						4.50 m

gt:								 906

MACHINERY/ PROPULSION

main engines: 							2 x wartsila 6r32

total power:							4, 920 kw

propulsion:							2x controllable pitch conventional

bow thruster: 							1 x 300 kw

PERFORMANCE

bollard pull (ahead):						84 t

speed ahead (maximum) 						15 knots

speed (economic) 						11 knots

TANK CAPACITY

fuel oil 								410 m3

portable water 							280 m3

foam							 	20 m3

ACCOMODATION

accomodation, air conditioned for 18 persons

deck area

deck area (aft)						 200 m3

DECK EQUIPMENT – FORE

fore winch 						hydraulic

single drum – 100 t. brake load

line pull 10 t.@25m/min. on first layer

DECK EQUIPMENT – AFT

aft winch 						hydraulic

waterfall (2 drums) – 200 t.brake load

upper drum – line pull 100 t. @ 5m/min. on 1st layer

lower drum -– line pull 20 t. @ 5m/min. on 1st layer

stern roller 						4.5m. x 2.2m., 200swl

tugger winch (es)  					2 x karmoy 240 t.swl

towing pins 						2 x karmoy 240t.swl

stopper pins 						1 x karmoy 300 t.

OTHER EQUIPMENT

capacity fire-fighting monitors 				2 x 1200 m3/hr

crane 							3t swl. @ 14m. outreach